UNIT III

BIPOLAR JUNCTION TRANSISTOR (BJT) CIRCUITS

Topics: BJT structure and its operation with normal biasing, Transistor characteristics and parameters, DC operating point, Transistor as an amplifier, Transistor as a switch, Enhancement type MOSFET.

Objectives: The objective of this unit is to study various types of transistors, their configurations and applications.

Outcomes: At the end of this unit, the learner will be able to:

1. Understand the operation of a transistor.

2.  Understand the transistor configurations and their comparison.

3.  State applications of transistor.

4.  Compare BJT and MOSFET.

Pre-requisites:Learner should be familiar with the basics of diode and diode circuits before transistorcircuits
Introduction:

Advantages of Transistors:

Small in size, light weighted.

Operate at low voltages.

Higher efficiency.

Do not require any filament power.

Long life.

The  term  transistor  was  derived  from  the  words  “Transfer”  and  “Resistor”.  These two words describe the operation of transistor, which is the transfer of an input signal current from a low resistance circuit to a high resistance circuit. Transistor is a three terminal device. These terminals are Collector, Emitter and Base. The base terminal is called a control terminal. It is called a bipolar transistor because in Bipolar Junction Transistor the conduction takes place due to both the charge carriers i.e. electrons and holes. If the conduction is due to only one type of charge carriers i.e. majority charge carriers, then the transistor is called as “Unipolar” transistor.

1.1. Types of Transistor and Construction: 
There are two types of transistors.

1.1.1.  P-N-P Transistors. PNP transistor is constructed by sandwiching a thin N type semiconductor between two P type semiconductors. The construction and symbol of PNP transistor is shown in fig. 1.1.1(a) and fig. 1.1.1(b) respectively. As shown in symbol of PNP transistor of fig. 1.1.1(b), the emitter current flows into the emitter terminal. The majority  charge  carriers  in  PNP  are holes and minority charge carriers are electrons.

| Fig. 1.1.1(a) Construction of PNP Transistor | Fig. 1.1.1(b) symbol of PNP transistor . |
| --- | --- |

1.1.2.  N-P-N Transistors. NPN transistor is constructed by sandwiching a thin P type semiconductor between two N type semiconductors.  The construction and symbol of NPN transistor is shown in fig. 1.1.2(a) and fig. 1.1.2 (b) respectively. As shown in symbol of NPN transistor of fig. 1.1.2(b),    the emitter current flows out of the emitter terminal.

| Fig. 1.1.2(a) Construction of NPN Transistor | Fig. 1.1.2(b) Symbol of NPN transistor |
| --- | --- |

The majority charge carriers in NPN are electrons and minority charge carriers are holes.

1.1.3. Common important points about transistor construction:

Base is always thin and lightly doped layer.

Collector and Emitter layers are much wider than the base and are heavily doped as compared to base.

The Emitter is the most heavily doped layer because it has to emit electrons. Collector has wider space compared to emitter because it is needed to dissipate more heat.   The  arrow  is  always  placed  on  the  emitter  terminal  and  arrow  direction  indicates  the  direction  of conventional current flow of emitter current. NPN transistors are more popular than the PNP transistors. So we will consider NPN transistor in most of the explanation rather than PNP transistor.

As seen from the constructional diagram of transistors, we can see that transistor has two PN junctions, namely Base to Emitter Junction (JBE) or Emitter junction and Collector to Base junction (JCB) or Collector junction.

- PNP and NPN transistors are equivalent to two diodes connected back to back as shown in fig 1.1.3(a) and 1.1.3(b) respectively.

| Fig 1.1.3(b): Equivalent for PNP Transistor | Fig 1.1.3(b): Equivalent for NPN Transistor |
| --- | --- |

2. Transistor Biasing:

2.1. Unbiased Transistor:

A transistor is said to be unbiased when no external power supply is connected to it. Due to two PN junctions in transistor, depletion regions are formed at the JBE and JCB. The depletion region is not same on the two sides of the junctions as shown in fig. 2.1 for NPN transistor. Depletion region always penetrates more in lightly doped region i.e. base and less penetrates in heavily doped region i.e. collector and emitter.

Fig.2.1 Depletion region in an unbiased NPN transistor

2.2. Biased Transistor: A transistor is said to be biased when external power supply is connected to it. BJT is capable of operating in three different regions, depending on the biasing. The biasing condition for these three regions of operations is listed in the table 2.1

Table 2.1 Biasing conditions for different regions of operation

| Sr.No | Region of Operation | Base Emitter Junction | Collector Base Junction | Application |
| --- | --- | --- | --- | --- |
| 1 | Saturation region | Forward Biased | Forward Biased | As a Closed Switch |
| 2 | Cutoff region | Reverse Biased | Reverse Biased | As an Open Switch |
| 3 | Active region | Forward Biased | Reverse Biased | As an Amplifier |

Let us understand as how to bias the transistor in active region. As mentioned in the above table, in order to use transistor as an amplifier it must be operated in active region.

Fig.2.2 shows the biasing for the NPN transistors in the active region. External power supply is connected in such a way that JBE is made forward biased and JCB is  reverse  biased.  Figure also indicates the conventional flow  of currents IB,,IC and IE. From figure, we see that external supply VCC is made larger  than  supply VEE,  to  ensure  that collector to base junction is reverse biased. Note that the polarities for the  PNP  transistor  are  exactly  opposite  to  the  polarity  of  NPN transistor.

Fig.2.2 NPN transistor biasing in the active region

3.  Transistor Operation:

3.1. Operation of NPN Transistor:

We will study operation of NPN transistor in the active region. In order to operate transistor in active region, base emitter junction JBE is made forward biased and collector base junction JCB is reverse biased. The external supply is given as shown  in fig.3.1 to  make the transistor to  operate in active region. Due  to this biasing, depletion region for base emitter junction is narrow and is wider for collector base junction

As  the  base  emitter  junction  JBE is forward  biased,  it  reduces  the barrier  potential  and  causes  the majority  charge  carriers  i.e. electrons  to  flow  from  n  type emitter to p-type base region. This constitutes the emitter current IE. Few of  the  electrons  entering  into the  base  region  do  not  reach  the collector  region  because recombination  of  electrons  and holes takes place and they flow out of the base terminal as shown in fig. 3.1. As the base region is very thin and lightly doped, very few holes are available for recombination. Due to this only about 2% electrons will flow out of the base. This

Fig.3.1.Operation of NPN Transistor constitutes the base current IB.

IB is said to be just 2% of IE. The remaining 98% electrons cross the reverse biased collector junction and reach to the positive terminal of the external supply VCC. This constitutes the collector current IC. The collector current is much larger than the base current. The emitter current is thus the sum of collector current and base current. i.e.IE=IC+IB. IBis very small compared to IC so we can assume that the collector current is nearly equal to the emitter current. i.e. IC≈IE

3.2. Operation of PNP Transistor:

PNP  transistor  behaves  exactly  in  the same way as  the NPN  transistor, only difference  is,  the  majority  charge carriers  are  holes  and  minority  charge carriers are electrons. Here, in PNP as shown in fig. 3.2 holes are emitted from the p-type emitter region into the n-type base region.  Base region is thin and lightly doped and so very few electrons are  available  for  recombination. Therefore  about  2%  of  total  emitted holes will flow out of the base terminal and  remaining  98%  are  collected  by collector region.

Fig. 3.2.Operation of PNP Transistor

4.  Transistor Configurations: In transistor, depending on which terminal is made common between input and output port, there are three possible configurations of the transistor. They are: Common Base (CB) Configuration, Common Emitter (CE) Configuration, and Common Collector (CC) Configuration. We will study these considering the active region of operation of Transistor and considering NPN transistor only , because it is the most popular transistor in the  market .Let us start with the CB configuration .

4.1. Common Base Configuration (CB): In  common  base  configuration,  base  terminal  is  made  common between  input  port  and  output  port.  The input is applied between emitter  and  base.  So the input voltage  is  VBE and  input  current is IE. Output is taken between collector and base. So the output voltage is VCB and output current is IC.

Fig 4.1.1 Common Base Configuration for NPN transistor Components in Collector Current :   IC(INJ) and ICBO

The collector current IC in common base configuration is given by,

IC= IC(INJ) + ICBO                                                                                                               (4.1.1)

Where, IC(INJ)   is called as injected collector current and is due to the number of electrons crossing the collector base  junction  and ICBO is  reverse  saturation  current flowing  through  the  transistor  due  to  minority  carriers between collector and base when the emitter terminal is kept open.

This current flows due to reverse biased collector base junction.

ICBO is negligible as compared to IC(INJ)    and so we can neglect it.

IC= IC(INJ)   ………Practically                                                                                        (4.1.2)

IC=ICBO……….. When emitter terminal is kept open                                                   (4.1.3)

These current IC(INJ)and ICBO are indicated in fig. 4.1.2(a) and fig. 4.1.2(b) respectively.

| Fig 4.1.2(a) Operation of Normal transistor for IC(INJ) | Fig 4.1.2(b) With Emitter terminal open |
| --- | --- |

Current amplification factor or current gain(αdc):

Current gain is the ratio of output current to  the input current. As seen  just now, we have IC(INJ) is output current and IE is the input current.

So, αdc=IC(INJ)IE                                                                                                           (4.1.4)

As value of IC(INJ) is less than IE, value of αdc will be always less than 1.Depending on the base thickness value of  αdc ranges from 0.95 to 0.995. If thickness of base is large, recombination at base will be more and so smaller is IC and hence smaller the value of αdc. From, equation (4.1.4),

we can write, IC(INJ=αdc  x  IE (4.1.5)

Equation number 4.1.1 reduces to,

IC=  αdc  X IE +ICBO                                                                               (4.1.6)

Neglecting ICBO above equation reduces to,   IC≈  αdc  X IE                                     (4.1.7)

Current gain =αdcICIE                                                                                                  (4.1.8)

Expression for IB: We have,

IE=IC+IB

Substituting the value of IC, we get,

IE=  αdc  X IE +ICBO    +IB

So, IB=1-αdcIE-ICBO

Neglecting ICBO,

we get, IB=1-αdcIE                                                                                         (4.1.9)

4.1.1. Characteristics of Transistor:

The characteristics of transistor help us to understand its behaviour. Input characteristics is basically a plot of input current verses input voltage and output characteristics is a plot of output current verses output voltage.

Input and output characteristics of CB configuration:

| Input characteristics | Output characteristics: |
| --- | --- |
| Input characteristics of CB configuration is a plot of input current IE verses input voltage VBE, keeping output voltage VCB constant. VBEis plotted on the X-axis and emitter current IE is plotted on the Y-axis as shown in fig. 4.1.1.1. Fig 4.1.1.1Input Characteristics The input characteristics is similar to the forward V-I characteristics of diode. Up to cut in voltage, the emitter current is negligible but after cut in voltage it increases rapidly with a very small increase in the input voltage VBE. ri=∆ VBE∆ IE and ri=1Slope =∆IE∆VBE Change in emitter current (∆IE) is very large compared to a small change in input voltage (∆ VBE) so the input resistance (ri) is small. Early Effect or Base width modulation: we can observe that, the emitter current IEincrease slightly with increase in the output voltage VCB. When VCB i.e. reverse bias voltage increase, width of depletion region increases which in turn reduces the electrical base width. This effect is called Early effect and as the width of base is changing it is called base width modulation | An output characteristic of CB configuration is a plot of output current IC verses output voltage VCB, keeping input current IE constant. VCBis plotted on the X-axis and collector current IC is plotted on the Y-axis as shown in fig. 4.1.1.3. Fig 4.1.1.3 Output Characteristics Cutoff region: Here JBE and JCB , both the junctions are in reverse biased. The region below the curve for IE=0is called cutoff region. This is because when input current IE is zero, the transistor will remain in off state and is nothing but emitter terminal open. ICBO is very small. Saturation region: Here JBE and JCB, both the junctions are in forward biased therefore the saturation region corresponds to negative values of VCBas shown in fig. 4.1.1.3. This is because, in CB configuration base is a common terminal and so voltages are measured with respect to this common point (reference point or ground point). In order to forward bias collector to base junction, collector terminal must be at .7V. Hence saturation region corresponds to negative values of VCB. Output current IC in this region increases exponentially with increase in VCB . The slope of the output characteristics in this region is large. i.e. Ro=1Slope= ∆IC∆VCB So the output dynamic resistance (ro)has a small value. This indicates that the voltage drop across the transistor VCE is mall in this region. Active region: Here ,output current (IC) is almost equal to the input current (IE) i.e. IC=IE irrespective of the variation in output voltage VCB .This is the reason that transistor is called a “Current controlled “ore “Current operated device and can be used as an “Constant Current Source “application in this region of operation. |

4.2. Common Emitter Configuration (CE)

4.2.1. Introduction

In common Emitter configuration, emitter terminal is made common between input port and output port. The input is applied between base and emitter. So the input voltage is VBE and input current is IB. Output is taken between collector and emitter. So the output voltage is VCE and output current is IC

Fig 4.2.1 Common Emitter Configuration for NPN transistor

Components in Collector Current:

- ICEO:

We will see here , the reverse saturation current in CE configuration and derive its equation.

From equation (4.1.6) ,

Ic=αdc ×IE  +ICBO

Ic=βdc ×IB  +1+βdcICBO i.e

Ic=βdc ×IB+ICEO

..where  ICEO=(1+βdc)ICBO

From this we can say that ICEO is the reverse saturation current for CE configuration. ICEO is large but very small as compared to βdc IB, so neglecting this ICEO , above equation of IC reduces to,

βdc=IcIB=Output currentInput current

This βdc is called current amplification factor or current gain for common emitter configuration.

Relationship between αdc and βdc

Βdc=αdc1-αdc, αdc=βdc1-βdc     βdc is much higher than αdc because, if αdc=0.99 then putting this value in above equation we will get βdc=99 .

Important points of ICEO:

We know that ICEO=(1+βdc)ICBO and value of βdc is much greater than 1, so. ICEO≫ ICBO .

Also we know that Ic=βdc ×IB+ICEO, if we substitute IB=0 ,then IC=ICEO.i.e. if base is open, then the collector current is equal to ICEO. It shows that ICEO is a reverse saturation current in CE configuration, which flows from collector to emitter terminal when base is kept open as shown in Fig. 4.2.2.

Fig.4.2.2.ICEOWith base terminal open

ICEO increases with increase in temperature. ICEO flows in the same direction as that of IC. Thus,  IC  increases with increases with increase in temperature even if IB is constant. This is known as thermal instability and thus in CE configuration, thermal stabilizing circuit is needed.

4.2.2. Input and output characteristics of CE configuration:

| Input characteristics: Input characteristics of CE configuration is a plot of input current IB verses input voltage VBE, keeping output voltage VCE constant. VBE is plotted on the X-axis and base current IB is plotted on the Y-axis as shown in fig. 4.2.2.1. The input characteristics is similar to the forward V-I characteristics of diode. Up to cut in voltage, the base current is negligible but after cut in voltage it increases rapidly. Here,. ro=1Slope=∆IB∆VBE As observed in the characteristics shown in fig. 4.2.2.1, change in base current(∆IB) is very large as compared to a small change in input voltage (∆VBE) so the input resistance( ri) is small in CE configuration but not as small as that of of CB configuration. It means it gives medium range of input resistance. As shown in Fig. 4.2.2.1., we can also observe the effect of VCE. It shows that, at constant VBE, IB decreases as VCE increases. Fig 4.2.2.1. Input Characteristics of transistor in CE configuration Fig 4.2.2.3 Collector base junction is forward biased. | Output characteristics : An output characteristics of CE configuration is a plot of of output current IC verses output voltage VCE, keeping input current IB constant. VCE is plotted on the X-axis and collector current IC is plotted on the Y-axis as shown in fig. 4.2.2.2. Cutoff region: Here JBE and JCB , both the junctions are in reverse biased. The region below the curve for IB=0 is called cutoff region. This is because when input current IB=0 is zero, the transistor will remain in off state and is nothing but base terminal open. ICEO is very small but high as compared to ICBO. Saturation region: Here, both the junctions are in forward biased. Therefore the Collector to base junction can be forward biased if and only if VCE drops down to about 0.2V. When this drops down to 0.2V then VBE=0.7 will be forward bias the collector to base junction. This is indicated in fig. 4.2.2.3. Typically the saturation voltage of transistor VCE(sat) is between 0.1 to 0.3volts. Output current IC in this region increases rapidly with increase in VCBas shown in fig. 4.2.2.2.In this region IC is is independent of IB and VCE .So, here the transistor is considered to be a resistor of very small value and hence used as a switch in this region is large for a small value of ∆VCE Active region: In active region base to emitter junction is forward biased and collector to base junction is reverse biased. Here, the output current IC increases slightly with increase in output voltage VCE at a constant base current. Here IC is largely dependent on IB. At a fixed value of VCE if IB is increased then IC is substantially increased. This is because IC=β IB is possible in active region of operation. The output resistance in this region is large because ∆IC in this region is very small.i.e. ro=1Slope=∆IC∆VCE Fig:4.2.2.2. Output Characteristics of transistor in CE configuration |
| --- | --- |

Typical junction voltages:

Table 4.2.2.shows the values of junction voltages for the  base to emitter  junction and collector to base junction  for NPN transistor at 25̊C

4.3. Common Collector Configuration (CC):

4.3.1 Introduction

In common Collector Configuration, collector terminal is made common between input port and output port. Input is applied between base and collector. So the input voltage is VBC and input current is IB. Output is taken between collector and emitter .So the output voltage is VEC  and output current is IE. CC configuration is also known as “Emitter Follower” configuration. Fig 4.3.1. Shows Common collector configuration for NPN transistor

Fig 4.3.1.  Common collector configuration for NPN transistor

The common collector configuration is basically same as the common emitter configuration; the only difference is that the load is connected in the emitter terminal rather than in the collector terminal.

Current relations in CC configuration:

Current gain in CC configuration (γ):

This ratio IEIB is called current gain and is denoted by γ (Gamma).

i.e. γ=IEIB=IC+IBIB=βdc+1

From this we conclude that γ(current gain) is a very high value compared to αdc and βdc .

The relation between γ and αdc i.e.γ=11-αdc

4.3.2. Input and output characteristics of CC configuration:

| Input characteristics: Input characteristics of CC configuration is a plot of input current IB verses input voltage VCB, keeping output voltage VEC constant. VBC is plotted on the X-axis and base current IB is plotted on the Y-axis as shown in fig. 4.3.2. The characteristics is shown for VEC=1V and at a constant output voltage of VBC=1.5V Fig 4.3.2 Input Characteristics Here, the base emitter junction is not forward biased up to VBC=1.5V; so the base current is indicated as zero up to VBC=1.5V . Once the base emitter junction is more and more forward biased then the base current increases rapidly as the VBC is increased beyond 1.5V. In order to forward bias the base emitter junction, it is necessary that the input voltage VBC must be higher than the output voltage VEC.This indicates that the voltage gain in CC configuration is always less than 1. | Output characteristics: An output characteristic of CC configuration is a plot of output current IE verses output voltage VEC, keeping input current IB constant. VEC is plotted on the X-axis and collector current IE is plotted on the Y-axis as shown in fig. 4.3.3. Fig 4.3.3 Output Characteristics From the characteristics it is clear that output characteristics of CC configuration are similar to those for CE configuration. Only the difference is that the saturation region has all positive values as compared to the negative saturation region in CE configuration. The CC configuration is used for the impedance matching because it has high input impedance and low output impedance. This is exactly opposite to that of CB configuration. |
| --- | --- |

4.4. Comparison of Configurations:

Comparison of all three configurations is indicated in table 4.4.1.

Table 4.4.1Comparison of CB, CE and CC configuration

| Sr.No. | Name of the Parameter | CB | CE | CC |
| --- | --- | --- | --- | --- |
| 1 | Common terminal | Base | Emitter | Collector |
| 2 | Input Current | IE | IB | IB |
| 3 | Output Current | IC | IC | IE |
| 4 | Input voltage | VBE | VBE | VBC |
| 5 | Output voltage | VCB | VCE | VCE |
| 6 | Phase shift between Input and output | 0̊ | 180̊ | 0̊ |
| 7 | Current gain notation | αdc=ICIE | βdc=ICIB | Υ=IEIB |
| 8 | Current gain | αdc (Less than 1) | βdc Always High | Very High |
| 9 | Voltage gain | Medium | Medium | Less than unity |
| 10 | Input resistance | Very low (20Ω) | Moderate (1kΩ) | Very High (500KΩ) |
| 11 | Output resistance | Very high (1MΩ) | High (10KΩ) | Very Low (50Ω) |
| 12 | Applications | As a Preamplifier | Audio Amplifier | For impedance Matching |

We  can  conclude  from  this  comparison  that  CE  configuration  is  the  best  among  three  configurations.  The reason behind this is it has high voltage gain and current gain. Due to high voltage gain and current gain, power gain is also high. CE configuration has moderate values of input resistance and output resistance. So just by connecting transistor in CE stage, automatic impedance matching can takes place.

5. DC load line:

DC load line is nothing but biasing the transistor by giving external dc power supply and plotting a load line on the characteristics of CE configuration. Let  us now study the procedure to plot this load line under DC biasing and its importance. Consider the common emitter configuration shown in Fig. 5.1.

Applying  KVL  to  the  collector  side  as  indicated  in  Fig.5.1  we  can     write,

VCC-ICRC-VCE=0

Rearranging this equation in terms of IC we get,

IC=-1RCVCE+VCCRC

Compare above equation with general equation of a straight line i.e.

y=mx+C

Fig 5.1Common Emitter configuration

Comparison gives,   y=IC, x=VCE ,m=-1RCand constant C=VCCRC

This shows that the equation  IC=-1RCVCE+VCCRC represents a straight line and this straight line is called as DC load line. Here , m is the slope of the line and is nothing but  m=-1RC,where RC is the collector resistor and output is usually measured at this terminal i.e. collector terminal.

This resistor  at   the  output  terminal  is  as  good  as  a    load resistor and so it is called as a LOAD LINE.

If we substitute VCE=0 in equation IC=-1RCVCE+VCCRCthen we get IC=VCCRC  which is ICmax or point “A” in the plot shown in Fig. 5.2. If we substitute  IC=0 in equation IC=-1RCVCE+VCCRC   then we get VCE=VCC which is a point “B” in the plot shown in Fig. 5.2.

Fig 5.2 DC load line Plotted with Q point

Quiescent Point (Q Point) or DC operating point: The term quiescent means quiet, still or inactive. Due to this Q point is also called as “Operating point” or “bias point”. Q point is the point on the DC load line which represents the dc current flowing through a transistor (ICQ)and the  voltage across it (VCEQ) ,  when no  ac signal  is  applied at  the input.  It shows the  dc biasing condition coordinates of Q point i.e. (VCEQ, ICQ). The position of Q point on the DC load line decides the application of the transistor. For Example, if transistor is to be used as an amplifier then Q point should be placed exactly at the centre of the DC load line to avoid any distortion in the amplified output waveform.

Stability of Q Point:

The factors which affect the stability of Q point are:

- Change in the value of β dc

- Change in temperature

- Variation in the parameters of one transistor to the other transistor

6. Biasing Circuits:

Biasing circuits are used to stabilize the Q point. Basically there are three types of biasing circuits.

They are:

Fixed bias circuit

Collector to base bias circuit

Voltage divider bias circuit or self bias circuit

Out of these biasing circuits, Voltage divider bias circuit or self bias circuit is the best to provide stability of Q point on the DC load line.

8. Transistor Application:

These are two major applications of transistor. They are transistor as a switch and transistor as an amplifier. Let us study them one by one.

8.1. Trasnistor as a switch:

Transistor can be used as a switch when it is biased to operate in saturation region (ON state) and cut off region (OFF state )

| Transistor in cut off region (OFF state /open Switch) When transistor is operated in cut off region both the junctions are in reverse biased and a very small reverse saturation current flows through the transistor. The input voltage Vin is zero as shown in Fig. 8.1(a). So IB=0 and ultimately IC=0. If we apply a KVL to the collector circuit of Fig. 8.1(a) We get, Vcc-IcRc-VCE=0 But as Ic=0, VCE=VCC. This shows that the voltage drop across the transistor i.e. VCE is high. Transistor offers a very high resistance to the flow of current through it, ideally infinite as collector current is zero. i.e. IC = 0 so we can say that in cut off region transistor is equivalent to an open switch or said to be in OFF state as shown in Fig.8.1(b). Fig 8.1.(a)Cut off region Fig 8.1.(b)Equivalent circuit | Close switch When transistor is operated in saturation region both the junctions Vin is applied at the base of the transistor as shown in Fig. 8.2(a). Base resistance RB is selected such that a large amount of IB flows. This will saturate the transistor. In order to saturate the transistor following condition should be satisfied. IB≥IC(sat)βdcWhen transistor is saturated, the voltage drops across the transistor i.e. VCE is very small. It offers a very less resistance, ideally zero and thus maximum collector current can flow through it. This indicates that transistor acts as a closed switch or is said to be in ON state as shown in Fig. 8.2(b) Fig 8.2.(a)Saturation region Fig 8.1.(b)Equivalent circuit |
| --- | --- |

Important Points to be noted:

When the input is low (0 Volts), the transistor is OFF, acts as an open switch and output voltage VCE=VCC.   When  the  input  is  high,  the  transistor  is  ON, acts  as  a  close  switch  and  output  voltage VCE=VCE(sat)≈0V.

Input and output voltage indicates that output of the transistor is 180̊ out of phase with the input. As transistor is  connected in  CE configuration  we  can say that  there  is  a phase  shift  of  180̊ between  input  and  output  voltage  in  CE configuration.

Ideally if input applied is a square wave then output will be a inverted square wave.

Practically switching between ON state to OFF state or OFF state to ON state does not take place instantly so some delays are introduced and are indicated as shown in Fig. 8.3

Fig. 8.3 Input and Output waveforms for Transistor switching

8.2. Transistor as an amplifier:

Amplification is a process of increasing or magnifying the strength of the input

8.2.1. Transistor as a voltage amplifier:

A voltage amplifier is basically designed to amplify the input voltage. Referring the simplified circuit of a BJT as shown in Fig.8.2.1,

The transistor is connected in CE configuration.  The  output  voltage  is  measured  at  the  collector  with  respect  to ground. VO=VCE.

If there is a small change in input voltage Vin then base current IB also changes and is represented by, ∆IB≈∆VinRB

As input current IB changes, the corresponding collector current changes and is          Fig. 8.2.1 Simplified circuit of BJT     given by,

∆Ic=βdc×∆VinRB

The corresponding change in output voltage Vo is given by,

∆Vo=∆Ic×RL

Substituting the value of ∆IC  we get,   ∆Vo=βdc×∆VinRB×RL Rearranging properly, ∆Vo=βdc RLRB×∆Vin

This equation indicates that for a small change in Vin we get a large change in Vo by a multiplication factor of  βdc RLRB    and voltage amplification takes place. Hence BJT can be used as a voltage amplifier.

8.2.2. Transistor as a current amplifier:

Transistor can be used as a current amplifier only in CE and CC configuration. Because we know that,   Current gain in CE configuration is =ICIB=βdc    and current gain in CC configuration is =IEIB=1+βdc.

As βdc  is much larger than 1, we can say that current gain in CE and CC configuration is large. So transistor connected in CE and CC configuration can be used as a current amplifier. Transistor can’t be used as a current amplifier in CB configuration, because the current gain αdc  is less than unity.

8.2.3. Single stage RC coupled CE amplifier: The  circuit  diagram  of  single  stage  RC  coupled  CE amplifier is shown in Fig. 8.2.3.1.

The capacitors C1 and C2   are called  as  coupling  capacitors.  The  load  resistor  is coupled to the amplifier through the coupling capacitor, this amplifier is called as RC coupled amplifier. As the transistor is connected in CE configuration it is called as CE amplifier.

Self biasing and active region of operation of transistor is provided to the circuit using R1,R2 and RE.

The coupling capacitor C1 is used couple the ac input voltage Vin to the base of the transistor .It is used to block dc component at the input. The output coupling capacitor C2 is used to couple the amplifier output to the load resistance or to the next stage of the amplifier. It is used to pass only AC part of the amplified signal to the load. The capacitor CE is connected in parallel with the emitter resistor RE and is called as emitter bypass capacitor. This capacitor offers a very low reactance to the amplified AC signal so the emitter resistor RE gets bypassed through CE

for only AC signals. This will increase the voltage gain of the amplifier. We have already studied how the BJT provides voltage amplification in CE configuration at the last point under 8.2.1.

Selection of Q point:

The criteria for selecting Q point to operate the transistor as an amplifier is:   The Q point should not be too close to the saturation region.

The Q point should not be too close to the cutoff region.

The Q point should be located at the center of the DC load line. This will ensure that the amplified signal is an exact replication of the input signal. The first two criteria should be satisfied in order to avoid any waveform distortion in the amplified output signal.

Fig. 8.2.3.2 (a),Fig. 8.2.3.2 (b) and Fig. 8.2.3.2 (c) are indicated for the three criteria mentioned above i.e. effect on the output due to shifting of Q point too close to the saturation region is indicated in Fig. 8.2.3.2 (a).Here, as seen from the figure the negative part of the output voltage waveform gets clipped off. The effect on the output due to shifting of Q point too close to the cutoff region is indicated in Fig. 8.2.3.2 (b). As seen from the figure the positive part of the output voltage waveform gets clipped off. The exact replication of the input signal at the output i.e. proper amplification is indicated in Fig 8.2.3.2.(c)

| (a) Effect of Q point close to the saturation region | (b) Effect of Q point close to the Cut off region | (a) Proper amplification |
| --- | --- | --- |